WSU-COS 107 Practical 8: Upgrade Proposal Sheet

Use the compatibility decision tree (Figure 31) and your asset record to judge each proposed part for YOUR bench PC. Record Yes/No for each test in Table H3 of your workbook.

| Bench number |  |
| --- | --- |

Proposal 1: Memory upgrade

| Part | 16 GB kit (2 × 8 GB) desktop memory |
| --- | --- |
| Type | DDR4-3200 UDIMM, 288-pin, 1.2 V |
| Reason for proposal | The PC slows down when many browser tabs are open. |
| Your decision (Compatible / Not compatible / Needs another change) |  |
| Evidence from your asset record (socket, RAM type, PSU wattage) |  |

Proposal 2: Processor upgrade

| Part | Intel Core i5-12400 processor |
| --- | --- |
| Socket | LGA1700 |
| TDP | 65 W |
| Reason for proposal | Faster compiling and virtual machines. |
| Your decision (Compatible / Not compatible / Needs another change) |  |
| Evidence from your asset record (socket, RAM type, PSU wattage) |  |

Proposal 3: Graphics card

| Part | Desktop graphics card, PCIe 4.0 x16 |
| --- | --- |
| Typical board power | 220 W |
| Manufacturer's recommended PSU | 650 W, with one 8-pin PCIe power connector |
| Reason for proposal | Video editing for a student project. |
| Your decision (Compatible / Not compatible / Needs another change) |  |
| Evidence from your asset record (socket, RAM type, PSU wattage) |  |

Instructor note: these proposals assume a typical DDR4 desktop with an LGA1200 (10th/11th generation Intel) board and a 300 to 400 W PSU. If your lab PCs differ, edit the parts so that one is compatible, one has the wrong socket or RAM type, and one is limited by the PSU.